| PLM Lipid composition | KCl, M | Number of experiments | Gm, pS 10 μM VBIT-4 | Gm, pS 20 μM VBIT-4 | Gm, pS 50 μM VBIT-4 |
| --- | --- | --- | --- | --- | --- |
| DPhPC | 0.15 | 2 | 30 ; 230 | 540 |  |
| PLE | 0.15 | 6 | 180 ; 590 | 3200 | 850 ; 1110 |
| PLE | 1 | 3 | 29 ; 200 | 316 ; 97 | 3260 |
| PC:PE:chol | 0.15 | 3 | 54 ; 45 | 670 ; 806 | 319; 3630 |

Supplemental Table 1. VBIT-4 induces conductance in planar lipid membranes (PLMs) of different lipid compositions. Summary of electrophysiology experiments performed on PLMs of three lipid compositions – DPhPC, PLE, and DOPC:DOPE:chol (60:32.5:7.5) - in  membrane-bathing solutions of 0.15 or 1.0 M KCl, buffered with 5 mM HEPES at pH 7.4. The effect of VBIT-4 on membrane conductance (Gm) was measured ~15 min after the addition of VBIT-4 to one side (cis) of the membrane at an applied voltage of 100 mV. Each Gm corresponds to the highest conductance measured at a given VBIT-4 concentration; those shown in the same color in each row correspond to the Gm values measured on the same membrane. Gm varies significantly across experiments, even under the same experimental conditions. Some PLMs ruptured upon the addition of 10 µM VBIT-4, while others remained stable until higher concentrations. Most membranes ruptured at ~30 µM of VBIT-4, although a subset resisted up to 50 µM VBIT-4. Notably, 50 µM was the highest concentration of VBIT-4 at which PLMs remained intact for at least 15 min after VBIT-4 addition at an applied voltage of 100 mV. Only the PLMs with conductance < 1 pS in the control before VBIT-4 addition were used in all experiments.