|  | c | k | α1 | α2 | α3 | α4 | α5 | α6 | α7 | α8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type I | -2.6E-01 | -1.7E+00 | 8.6E-01 | -5.3E+01 | 6.0E+03 | -2.77E+05 | 7.3E+06 | -8.9E+07 | 2.5E+08 | 2.2E+09 |
| Type II | 1.0E+03 | -2.4E+03 | -1.5E+00 | 2.9E+01 | -4.6E+01 | -7.69E+03 | 2.0E+05 | 7.8E+06 | -1.7E+08 | -3.9E+08 |
| Type III | -5.0E-01 | 8.2E+00 | 1.5E+00 | -1.4E+02 | 3.7E+04 | -3.92E+06 | 2.2E+08 | -5.6E+09 | 3.3E+10 | 6.3E+11 |
| Type IV | -2.4E-01 | -1.2E+00 | 9.2E-01 | -1.3E+02 | 4.4E+04 | -4.28E+06 | 2.2E+08 | -5.6E+09 | 3.5E+10 | 4.9E+11 |

Supplementary Table 1. Parameters for the fabrication of corrective lenses. Coefficients used in equation (1) (see Materials and methods) for the aspherical corrective lenses used in type I-IV eFOV-microendoscopes.

| Type | Focal length in uncorrected microendoscopes (µm) | Focal length in eFOV- microendoscopes (µm) |
| --- | --- | --- |
| I | 190 | 188 |
| II | 194 | 201 |
| III | 195 | 172 |
| IV | 237 | 226 |

Supplementary Table 2. Simulated focal length in uncorrected and corrected microendoscopes.

| Type | On axis FWHMx,y (µm) n ≥ 8 | On axis FWHMz (µm) n ≥ 8 | Effective FOV radius (µm) | Fold increase in FOV area |
| --- | --- | --- | --- | --- |
| I | uncor.: 0.91 ± 0.01 cor.: 0.94 ± 0.02 p = 0.1607 | uncor.: 9.54 ± 0.18 cor.: 8.85 ± 0.12 p = 0.0026 | uncor.: 74 cor.: 133 | 3.2 |
| II | uncor.: 0.80 ± 0.01 cor.: 0.87 ± 0,02 p = 0.0064 | uncor.: 8.91 ± 0.15 cor.: 8.87 ± 0,11 p = 0.7916 | uncor.: 51 cor.: 156 | 9.4 |
| III | uncor.: 0.88 ± 0.02 cor.: 0.89 ± 0.00 p = 0.4298 | uncor.: 7.72 ± 0.07 cor.: 7.45 ± 0.16 p = 0.1861 | uncor.: 62 cor.: 154 | 6.2 |
| IV | uncor.: 0.78 ± 0.02 cor.: 0.89 ± 0.03 p = 0.0041 | uncor.: 8.86 ± 0.24 cor.: 8.32 ± 0.29 p = 0.1583 | uncor.: 34 cor.: 69 | 4.1 |

Supplementary Table 3. Spatial resolution and effective FOV of eFOV-microendoscopic probes. Value are reported as average ± sem. For statistical comparison of uncorrected (uncor.) vs corrected (cor.) microendoscopes, Student’s t-test was used.

| Pupil diameter range | 0-0.1 | 0.1-0.2 | 0.2-0.3 | 0.3-0.4 | 0.4-0.5 | 0.5-0.6 | 0.6-0.7 | 0.7-0.8 | 0.8-0.9 | 0.9-1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Q vs W | p = 1 | p = 1 | p = 1 | p = 1 | p = 1 | p = 1 | p = 0.7 | p = 1 | p = 1 | p = 1 |
| Q vs WL | p = 1 | p = 0.04 | p = 7E-5 | p = 2E-6 | p = 2E-4 | p = 0.7 | p = 1 | p = 0.009 | p = 2E-6 | p = 2E-6 |
| W vs WL | p = 1 | p = 0.8 | p = 0.9 | p = 0.002 | p = 0.04 | p = 0.2 | p = 1 | p = 0.8 | p = 2E-6 | p = 2E-6 |

Supplementary Table 4. Statistical comparisons of behavior state distributions as a function of pupil diameter. For the statistical comparison of Q, W, and WL state distributions in each range of pupil diameter, a two-way ANOVA with Tukey-Kramer post hoc correction was performed.