High-throughput Plasmodium falciparum hrp2 and hrp3 gene deletion typing by digital PCR to monitor malaria rapid diagnostic test efficacy

Claudia A. Vera-Arias et al.

Supplementary File 1

ddPCR assays

Instrument: BioRad QX200 Droplet Digital PCR System

Supermix from BioRad

All primers and probes at 10 µM

hrp2 exon 2 ddPCR

|  | [uL] |  | Cycling conditions |  |
| --- | --- | --- | --- | --- |
| SuperMix for Probes (no dUTP) | 11 |  |  |  |
| hrp2_Exon 2_fwd | 1.76 |  | 95° | 10 min |
| hrp2_Exon 2_rev | 1.76 |  | 94° | 30 sec |
| hrp2_Exon 2_Probe | 0.88 |  | 56° | 1 min => 44 x back to step 2 |
| tRNA_fwd | 0.44 |  | 98° | 10 min |
| tRNA_rev | 0.44 |  | 4° | hold |
| tRNA_Probe | 0.22 |  |  |  |
| H2O | 3.5 |  |  |  |
| DNA | 2 |  |  |  |

hrp2 exon 1 ddPCR

|  | [uL] |  | Cycling conditions |  |
| --- | --- | --- | --- | --- |
| SuperMix for Probes (no dUTP) | 11 |  |  |  |
| hrp2-exon1_fwd | 1.76 |  | 95° | 10 min |
| hrp2-exon1_rev | 1.76 |  | 94° | 30 sec |
| hrp2-exon1_Probe | 0.88 |  | 54° | 1 min => 44 x back to step 2 |
| tRNA_fwd | 0.44 |  | 98° | 10 min |
| tRNA_rev | 0.44 |  | 4° | hold |
| tRNA_Probe | 0.22 |  |  |  |
| H2O |  |  |  |  |
| DNA | 2 |  |  |  |

hrp3 ddPCR

|  | [uL] |  | Cycling conditions |  |
| --- | --- | --- | --- | --- |
| SuperMix for Probes (no dUTP) | 11 |  |  |  |
| hrp3_fwd | 0.66 |  | 95° | 10 min |
| hrp3_rev | 0.66 |  | 94° | 30 sec |
| hrp3_Probe | 0.44 |  | 55° | 1 min => 44 x back to step 2 |
| tRNA_fwd | 0.44 |  | 4° | hold |
| tRNA_rev | 0.44 |  |  |  |
| tRNA_Probe | 0.22 |  |  |  |
| H2O | 6.14 |  |  |  |
| DNA | 2 |  |  |  |

Nested PCR assays

Instrument: BioRad T100 Thermal Cycler

Polymerase, buffer, and dNTPs from Solis Biodyne

All primers at 10 µM

hrp2 nested PCR

| Primary PCR | [uL] |  | Cycling conditions |  |
| --- | --- | --- | --- | --- |
| Taq FirePol (Solis Biodyne) | 0.15 |  |  |  |
| Buffer B (Solis Biodyne) | 1.5 |  | 94° | 10 min |
| MgCl2 (Solis Biodyne) | 1.5 |  | 94° | 50 sec |
| dNTPs (Solis Biodyne) | 1.3 |  | 55° | 50 sec => 39x back to step 2 |
| Primer hrp2_F1 | 0.2 |  | 70° | 1 min |
| Primer hrp2_R1 | 0.2 |  | 70° | 5 min |
| H2O | 8.25 |  |  |  |
| DNA | 2 |  |  |  |
| Nested PCR | [uL] |  | Cycling conditions |  |
| Taq FirePol (Solis Biodyne) | 0.15 |  |  |  |
| Buffer B (Solis Biodyne) | 1.5 |  | 94° | 10 min |
| MgCl2 (Solis Biodyne) | 1.5 |  | 94° | 50 sec |
| dNTPs (Solis Biodyne) | 1.3 |  | 55° | 50 sec => 39x back to step 2 |
| Primer hrp2_F2 | 0.2 |  | 70° | 1 min |
| Primer hrp2_R1 | 0.2 |  | 70° | 5 min |
| H2O | 8.25 |  |  |  |
| DNA | 2 |  |  |  |

Primer sequences

Hrp2 F1		5′-CAAAAGGACTTAATTTAAATAAGAG-3′

Hrp2 R1		5′-AATAAATTTAATGGCGTAGGCA-3′

Hrp2 F2		5′-ATTATTACACGAAACTCAAGCAC-3′

msp2 nested PCR

Polymerase, buffer, and dNTPs from Solis Biodyne

All primers at 10 µM

| Primary PCR | [uL] |  | Cycling conditions |  |
| --- | --- | --- | --- | --- |
| ddH2O | 7.85 |  |  |  |
| Buffer B (Solis Biodyne) | 1.5 |  | 95˚ | 1 min |
| MgCl2 25 mM (Solis Biodyne) | 1.5 |  | 95˚ | 30 sec |
| dNTPs 2.5 mM (Solis Biodyne) | 1.2 |  | 55˚ | 30 sec |
| Primer msp2_S2-fw | 0.4 |  | 72˚ | 1 min => 34 x back to step 2 |
| Primer msp2_S3-rev | 0.4 |  | 72˚ | 5 min |
| Taq FirePol (Solis Biodyne) | 0.15 |  |  |  |
| DNA | 2 |  |  |  |
| Nested PCR | [uL] |  | Cycling conditions |  |
| ddH2O | 8.975 |  |  |  |
| Buffer B (Solis Biodyne) | 1.5 |  | 95˚ | 1 min |
| MgCl2 25 mM (Solis Biodyne) | 1.5 |  | 95˚ | 30 sec |
| dNTPs 2.5 mM (Solis Biodyne) | 1.2 |  | 55˚ | 30 sec |
| Primer M5_Fc27 (10 uM) | 0.15 |  | 72˚ | 1 min => 34 x back to step 2 |
| Primer N5 3D7 (10 uM) | 0.225 |  | 72˚ | 5 min |
| Primer S1-tail fw | 0.3 |  |  |  |
| Taq FirePol (Solis Biodyne) | 0.15 |  |  |  |
| Primary PCR as template | 1 |  |  |  |

Primer sequences

msp2_S2-fw		5’- GAAGGTAATTAAAACATTGTC-3’

msp2_S3-rev		5’- GAGGGATGTTGCTGCTCCACAG -3’

msp2_S1Tail-fw 		5’- GCTTATAATATGAGTATAAGGAGAA -3’

msp2_FC27 type_M5-rev	5’- GCATTGCCAGAACTTGAA -3’

msp2_3D7 type_N5 -rev	5’- VIC-CTGAAGAGGTACTGGTAGA -3’