|  | schematic of the method | name of the method | calculation of scanning speed |  | Tmeasurement (vmeasurement ) | Vmeasurement compared to chessboard scanning | ratio of collected photons compared to chessboard scanning (SNR2 ) | SNR2 × vgain compared to chessboard scanning |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AO SCANNING TECHNIQUES |  | 3D AO chessboard scanning | Ncell × Nline × Tpixel |  | 0.036 s (27.7 Hz) | 1 | 1 | 1 |
|  |  | AO point by point scanning of the entire volume | x × y × z × Tpixel |  | 1611.5 s (0.0006 Hz) | 1/44762 | 1 | 1/44762 |
|  |  | AO point by point scanning in layers containing cell somatas (19 layers) | x × y × Nz × Tpixel |  | 158.4 s (0.00631 Hz) | 1/4399 | 1 | 1/4399 |
| RESONANT SCANNING TECHNIQUES |  | Volume scanning with resonant mirror | x × y × z × T'pixel |  | 6.1 s (0.16 Hz) | 1/170 | 1/244 | 1/41506 |
|  |  | Multiple-layer scanning with resonant mirror and piezo (19 layers) | x × y × Nz × T'pixel |  | 0.98 s (1.04 Hz | 1/27 | 1/244 | 1/6654 |
|  | *Used parameters: Ncell = 120 (120 cells) x = 548 pixel, y = 507 pixel, z = 193 pixel (total scanning volume was: x = 689 µm, y = 639 µm, z = 580 µm) Nz=19 (19 z layers were used in volume scanning) |  |  | T’pixel = 0.11 µs, (pixel dwell time of resonant scanning, according to a f=16 kHz frequency and the x=548 pixel line resolution of the resonant scanner) Tpixel = 30 µs, (AO pixel dwell time) Nline= 10 (number of lines used to form a frame in chessboard scanning) |  |  |  |  |