| Hardware |  |  | Software |  |  |
| --- | --- | --- | --- | --- | --- |
| Microscope | Titan Krios |  | Data collection |  | EPU v 2.8.0 |
| Detector (mode) | K2 (counted) |  | Collection method |  | AFIS |
| Accelerating voltage | 300 keV |  |  |  |  |
| Spherical aberration | 2.7 |  |  |  |  |
| Data acquisition parameters |  |  |  |  |  |
| Apertures (C1, C2, C3) | 2000, 70, 2000 |  | Defocus range (µm, step size) |  | -1.2 to -2.7 (0.3) |
| Objective aperture | 100 |  | Dose (e/px/sec) |  | 7.75 |
| Energy filter slit (eV) | 20 |  | Dose (e/Å2/sec) |  | 11.5 |
| Illuminated area (µm) | 1.00 |  | Exposure time (sec) |  | 5 |
| Spot size | 6 |  | Total dose (e/Å2) |  | 57.5 |
| Tilt angle (°) | 0 |  | Dose fractions (#) |  | 40 |
| Nominal magnification | 165 000 x |  | Dose per fraction (e/Å2) |  | 1.44 |
| Pixel size (Å) | 0.82 |  |  |  |  |
| Data processing parameters |  |  |  |  |  |
|  |  | PrgB apo |  | PrgB ssDNA |  |
| Number of micrographs (total) |  | 2797 |  | 1670 |  |
| Initial particle number (subset 500 micrographs) |  | 170,826 |  |  |  |
| Initial particle number (total) |  | 1,186,745 |  | 714,102 |  |
| Final particle number (total) |  | 283,630 |  | 163,566 |  |
| Average map resolution (Å) |  | 8-10 |  | 9-11 |  |
| FSC threshold |  | 0.143 |  | 0.143 |  |