General steps for calcium signal analysis using the CScreen spatiotemporal screening method.

Method principle is described and discussed in Pham et al., NeuroImage 2020.

Dongdong Li, INSERM (dongdong.li@inserm.fr)

as of 2021-07-15

ImageJ FOR PRE-PROCESSING OF TIME LAPSE IMAGE STACKS

- 16bit image is used

- Background subtraction (must do): one way is to remove the minimum projection from the video

  - Subtract from the image stack its minimum value of the intensity. To get that value, just (i) go to “Analyze  Set Measurements”, choose “Min & max gray value”; Then, back to “Analyze  Measure”

  - Then, go to “Process  Math  Subtract” to substract the background from all images.

- Movie Shift correction (optional): could use ImageJ plugin ‘StackReg’; small shift could be corrected.

- Photobleaching correction (optional): looking at the whole-field mean fluorescence time course, if significant declining appears, could use the ImageJ plugin ‘EMBLtools  Bleach Correction’ to correct this declining. Normally, ‘Exponential Fit (Frame-wise)’ mode would work.

// *just note, correction of the global fluorescence bleaching/declining by exponential fit is also embedded in Matlab programs ‘EA0_...ExpRe.m’ and ‘AA0_...ExpRe.m’; if the bleaching is already corrected in ImageJ, no need to do it again in Matlab  just use ‘EA0_...nonRe.m’ or ‘AA0_...nonRe.m’ to import the movie.

- Binning the image to reduce size (optional): could use ImageJ to bin the image (Image  Transform  Bin, ‘X-2, Y-2 & average mode’ would be ok) // the reason is, the matlab program is still a prototype without being fully optimized; handling too large image files could be ‘Out of Memory’.

//*but note, depending on the physical size of the imaged objects, binning needs be carefully considered as it will reduce the effective pixel size.

- Save the pre-processed movie as raw data: ‘File  Save as  Raw Data’  name the file as ‘GFP_p.raw’

//* Remember to add the suffix “.raw” to the file name.	NOTE: if crop a subregion of the image stack to analyze, better to write down the dimension of the new stack, e.g., in a txt file put it in the corresponding analysis folder; or just mark this dimension in the folder name, etc; as this information is required for matlab programs

MATLAB INITIATION, IMAGEJ PRE-PROCESSING AND NOTES:

- To let Matlab find the programs, need use “Set path” to add (with subfolders) the folder that contains all the programs

- Matlab needs find the image data (i.e., GFP_p.raw file) and save all results in a Workspace folder. The directory pathway of this folder needs be indicated on the first line of each program after “cd”, i.e., to replace “D:\Dongdong\ZMwork” (which is the location in my ordi)

- Several notes on Matlab and results:

- The name of directory and files should avoid containing a ‘space’, or starting with a number

- The images automatically generated by Matlab can be exported as TIFF, which can then be inserted into any softwares for figure composition

//*BUT NOTE, THESE FIGURES ARE NOT GOOD FOR FINAL PRESENTATION. The figures containing traces scatter plots, and histogram bars, etc, have low adaptation to fit into small size. NEED TAKE THE ORIGINAL DATA POINTS THAT ARE CONTAINED IN RESULT FILES IN MATLAB WORKSPACE, to re-plot the figures in Igor (or other graphing softwares).

The exceptions are the pictures for the raster plot, correlation map, detected responsive regions, etc, which are generated by Matlab and automatically saved as TIFF file so can be directly used for figure generation.)

- The histograms shown during the program running are just from the single movie processed. The source data (used to construct the histogram) are saved in workspace, which can be pooled with data from other movies. The final histogram can be generated from the pooled data  using “B4_Histogram_Pooled_Data.m”, or directly by other softwares like Igor.

//*Concretely,

- pool all data points (derived from each movie) in Igor in a single column, then copy the pooled data to “B4_Histogram_Pooled_Data.m  pooled_data [ DATA COPIED IN HERE]”;

- Then, set the “bin_size = ## “, which is a percentage number that then multiplies the whole range of the data (i.e., bin_size*(max-min) ). Normaly, can start from 10%  (0.1), then adjust it abit to make the histogram properly displayed.

- Run the program (either by click on top menu, or press F5), histograms should be displayed

- Go to the Matlab workspace, find the three files: (1) B4_Pooled_Hist_X; (2) B4_Pooled_Hist_Y; (3) B4_Pooled_Cumulative_Y

- Then, drag these files to Igor, and properly name them, can use Igor to plot the histogram by B4_Pooled_Hist_Y vs. B4_Pooled_Hist_X, and the cumulative histogram by B4_Pooled_Cumulative_Y vs. B4_Pooled_Hist_X

In the text below:

“SHADOW IN GREY”  // means single Matlab programs

“BLUE NAMES” MEANS FIGURES GENERATED IN MATALB // can be used for direct view, but the trace&histogram graphs not good for final show

“RED NAMES” MEANS RAW DATA POINTS // for 1/ Replot figures in Igor; 2/ for pooling the data together in Igor; 3/can be also imported in other softwares

ANALYZE SPONTANEOUS ACTIVITY

Import Data into Matlab & calculate Time-lapse correlation of adjacent x-y frames (z, time axis), i.e., time-lapsed correlation between the (n)th to (n+1)th frames, so to know the global variability over time

- In Matlab main window, click the “open folder” icon, find the program “AA0_Time_lapse_Pearson_nonRe.m” and open it. Now, a program window appears.

- Note, need input

1/DATA DIMENSION (rd, cdcd, zd_o);

2/file_name = ’ ’ 	// can keep GFP_p.raw for all data

3/time_interval = 	// in s, check for different imagings

4/pixel_size = 		// in µm, depends on objective and additional magification

- Need input the dimension of the image stack: “rd” is the x dimension, “cdcd” is y dimension, “zd_o” is the number of frames (i.e., z dimension). Note the default file name is “GFP_p.raw”

- To run the program, can click the ‘Run’ button in the program window. Or, just press F5, the program will save and run.

- The appearance of a trace figure (“A0 Inter Frame Corr over time”) indicates the finishing of the program.

- The trace plot figure is saved as a TIFF image “A0_InterFrame_corr_overTime” in the workspace folder, and the source data (X/Y axis data points) saved as “A0_InterFrame_corrX/Y”. The source data files can be imported to Igor for fine plotting later on.

Response regions searched by spatial correlation of time courses of ministacks

(Generating the correlation map)

- In Matlab program window, open program “AB0_SpatialTemporal_Pixel_screening.m”.

- Normally, no need to change parameters

//*Note the variables “off_cdcd” and “off_rd”, which define the x-y size of the ‘mini block’ that is used to calculate spatial correlations across the image.  Now, the setting is 1 for each, defining a 2*2 mini block, z containing the temporal information.

- Run the program. It takes ~1-2 minutes to generate a correlation map in a 2D image (titled “(B0) PIXEL SCREENING DONE”).

- This image is then saved in the workspace folder as a tiff image “B0_Pixel_screening_corr_Map”

- ! NOTE: a raw file “B0_CorrMap_PIXEL_SCREENING” also saved in the workspace. You may save it. When you want to do some new processing from “B1start …”  “B3 …”, you can just copy this file together with the raw data to workspace and run B1start to B3, i.e., YOU DO NOT NEED RUN B0 again.

- Open “AB1start_Pixel_screening_filter.m” in the program window

- May first just use the default threshold of 0.14-0.2 //

//*Can also change it “thresd_percent = “// smaller number will give less responsive regions but maybe better separated; larger number give more responsive regions, but maybe less separated

- the threshold is chosen by keeping the pixels (in the correlation map) of the top percent (defined by “thresd_percent”) of correaltion values. This value is shown in the cumulative plot (“(B1) Cumulative of Corr Map”) of the distribution of all pixels of the correlation map. Picture saves as “B1_Cumulative_Corr_Map”.  The source data points are saved as “B1_Cumulative_CorrMapX/Y”

- Also, the histogram distribution of all pixels of the correlation map is shown in “B1_Histogram_Corr_Map”. The source data points are saved as “B1_Hist_CorrMapX/Y”.

- In B1start, need also define the minimal number of the data points in a response trace that are higher than mean+3*SD of the base line (line 26, “trisd_num”) // now, it’s set as “3”.

- A picture titled “(B1) REGION Thresholded&Filtered” finally pops out. // saved as a tiff image “ B1_REGION_Thresholded’percent’” //

(Get region map by excluding regions showing no obvious rises, and then get spatial info)

- Program “AB2_Pixel_Screening_spatial.m” //// This generates the final responsive region map, in which regions containing less than 10 pixels (defined by line 29 “P” value), and regions showing no obvious increases (i.e., > basal level + 3*SD) upon stimulation are removed;

- A base line is represented by a segment trace (length “fzero_end”-“fzero_start”+1) searched (running sliding, running step defined by “seg_incre”) across the basal imaging period (now, 1-60th frame, set by “resting_phase”)

- *the segment showing minimum mean taken as baseline // before, tested the segment with mini STD as basal line, but it is not good, as some negative values appear

- Here, a response should contain at least “trisd_num” consecutive data points that are >mean+3*SD of base line

- Click “Run”. A Black&White average image of the data stack pops out ‘(B2) POINT THE CENTER’, in which need click mouse to choose an approximate center position.

- The appearance of a final region map “(B2) REGIONS FOUND and ORDERED” indicates the finishing of the program. It’s saved as a TIFF image in the workspace folder with the same title.

- ALSO NOTE: AFTER THE APPERANCE OF A HISTOGRAM, another black white picture pops out ‘(B2) CHOOSE A REGION FOR BASAL DECLINE’ /// choose where, CAN FIRST CHECK IN IMAGEJ TO CHOOSE A SMALL REGION (10x10 pixel) on fluorophore loading area BUT NOT SHOWING Ca++ RISES, i.e., only show baseline declining (or if there is no declining, just choose a region showing no declining) /// this trace will be used to correct the declining, by letting the raw data time courses to divide it.

- Finally, a figure of declining trace appears.

- Two histograms are generated, one showing the spatial distribution of the distance-to-center of all response regions, and the distribution of region area (µm²). The images are saved in workspace as “B2_Distribution_Region_Area” //// Of note, the source data saved as ”B2_All_RegionArea”.

Get time-courses for all responsive regions

(Get temporal (-spatial) info for region responses)

- run “AB3_Pixel_Screening_temporal_SpontRe.m” to get temporal and spatio-temporal information about responses (regions), for spontaneous Ca++ responses.

- One parameter is the “skip_example = “ //// just for showing example traces; sometimes, there are too many traces, which can not be properly shown in Matlab figures; so can omit some, if set a number of 2 in here, meaning, only responses of region 2, 4, 6 … are shown, etc

- will get

- The time course of all responses (all regions), raster plot saved as a tiff image titled “B3_Raster_Plot_AllResponses” (of note: regions arranged according to their distance from the center, 1 means center, uppermost one showing the one most distant from the center.); source data saved as “B3_All_responses” that contains time courses for each region, and can be imported into Igor for plotting (BUT NEED USE ANOTHER IGOR ROUTINE …).

- Integral/minunte (s*dF/F0, i.e., area below the trace) of all responses. Its distribution image (the histogram) saved as “B3_Distribution_Response_Integral”; the source data saved as ”B3_All_Responses_Integral” that can be imported into Igor, pooled together with other cells’ data to plot the final histogram where a proper bin size should be settled.  // NOTE, THE HISTOGRAM SHOWN HERE IS A TEMPORARY DISPLAY, FOR FORMAL HISTOGRAM (especially for pooled data, e.g., INTEGRAL DATA FROM SEVERAL RECORDINGS), NEED use B4 to generate the X and Y data points for a histogram and then replot in Igor (see below).

- The distribution of the correlation between all responses (combinations). The histogram saved as a tiff image titled “B3_Distribution_All_InterResponse_Corr”, source data saved as “B3_All_Responses_InterCorr”

- Example traces. The traces saved a tiff image “B3_Example_Response_Traces”.

Note that the example traces are taken every ‘skip_example’ regions from all responses (regions). // as the example traces are marked by the N° of region, the source data points can be easily found in “B3_All_Responses” in Igor.

- Relation between the correlation of responses and their clustering degree.  The correlation and the inter-distance (a reflection of clustering) between all combinations of all regions are calculated and scatter is plotted, showing whether closer regions are more correlated or not. // Image saved as “B3_Scatter_Inter_Response_Corr_vs_Inter_Distance” //

source data (i.e., X, Y axis data points) saved as “B3_Inter_Response_Corr”, and “B3_Inter_Response_Distance”, respectively

ANALYZE AGONIST-EVOKED ACTIVITY

Time-lapse correlation of adjacent x-y frames (z, time axis)&CORRECTION OF PHOTOBLEACING, i.e., time-lapsed correlation between the (n)th to (n+1)th frames, so to know the variability over time

- In Matlab main window, click the “open folder” icon, find the program “EA0_Time_lapse_Pearson_nonRe.m” and open it. Now, a program window appears.

- Note, need input

1/DATA DIMENSION (rd, cdcd, zd_o);

2/file_name = ’ ’ 	// can keep GFP_p.raw for all data

3/time_interval = 	// in s, check for different imagings

4/pixel_size = 		// in µm, depends on objective and additional magification

For evoked responses, NEED ALSO define

5/ sti_start = 		// starting frame of stimulation, probably not be precise due to 					global perfusion; must be after switching on the stimulation 					channel

6/ sti_dur = 		// duration of global application/diffusion of the agonist

Then, the program will know which period is bath application					, and which period is the phase of bath washing

- Need input the dimension of the image stack: “rd” is the x dimension, “cdcd” is y dimension, “zd_o” is the number of frames (i.e., z dimension). Note the default file name is “GFP_p.raw”

- To run the program, can click the ‘Run’ button in the program window. Or, just press F5, the program will save and run.

- The appearance of a trace figure (“A0 Inter Frame Corr over time”) indicates the finishing of the program.

- The trace plot figure is saved as a TIFF image “A0_InterFrame_corr_overTime” in the workspace folder, and the source data (X/Y axis data points) saved as “A0_InterFrame_corrX/Y”. The source data files can be imported to Igor for fine plotting later on.

Response regions searched by spatial correlation of time courses of ministacks

(Generating the correlation map)

- In Matlab program window, open program “EB0_SpatialTemporal_Pixel_screening.m”.

- Normally, no need to change parameters

//*Note the variables “off_cdcd” and “off_rd”, which define the x-y size of the ‘mini block’ that is used to calculate spatial correlations across the image.  (Now, the setting is 1 for each, defining a 2*2 mini block, z containing the temporal information).

- Run the program. It takes ~1-2 minutes to generate a correlation map in a 2D image (titled “(B0) PIXEL SCREENING DONE”).

- This image is then saved in the workspace folder as a tiff image “B0_Pixel_screening_corr_Map”

- ! NOTE: a raw file “B0_CorrMap_PIXEL_SCREENING” also saved in the workspace. You may save it. When you want to do some new processing from “B1start …”  “B3 …”, you can just copy this file together with the raw data to workspace and run B1start to B3, i.e., YOU DO NOT NEED RUN B0 again.

- Open “EB1start_Pixel_screening_filter.m” in the program window

- May first just use the default threshold of 0.14 //

Can also change it “thresd_percent = “// smaller number will give less responsive regions but maybe better separated; larger number give more responsive regions, but maybe less separated

- the threshold is chosen by keeping the pixels (in the correlation map) of the top percent (defined by “thresd_percent”) of correaltion values. This value is shown in the cumulative plot (“(B1) Cumulative of Corr Map”) of the distribution of all pixels of the correlation map. Picture saves as “B1_Cumulative_Corr_Map”.  The source data points are saved as “B1_Cumulative_CorrMapX/Y”

- Also, the histogram distribution of all pixels of the correlation map is shown in “B1_Histogram_Corr_Map”. The source data points are saved as “B1_Hist_CorrMapX/Y”.

- In B1start, need also define the minimal number of the data points in a response trace that are higher than mean+3*SD of the base line (line 26, “trisd_num”) // now, it’s set as “3”.

- A picture titled “(B1) REGION Thresholded&Filtered” finally pops out. // saved as a tiff image “ B1_REGION_Thresholded’percent’” //

(Get region map by excluding regions showing no obvious rises, and then get spatial info)

- Program “EB2_Pixel_Screening_spatial.m” //// This generates the final responsive region map, in which regions containing less than 10 pixels (defined by line 29 “P” value), and regions showing no obvious increases (i.e., > basal level + 3*SD) upon stimulation are removed;

- A base line is represented by a segment trace (length “fzero_end”-“fzero_start”+1) searched (running sliding, running step defined by “seg_incre”) across the basal imaging period (now, 1-60th frame, set by “resting_phase”)

- *the segment showing minimum mean taken as baseline // before, tested the segment with mini STD as basal line, but it is not good, as some negative values appear

- Here, a response should contain at least “trisd_num” consecutive data points that are >mean+3*SD of base line

- Click “Run”. A Black&White average image of the data stack pops out ‘(B2) POINT THE CENTER’, in which need click mouse to choose an approximate center position.

- The appearance of a final region map “(B2) REGIONS FOUND and ORDERED” indicates the finishing of the program. It’s saved as a TIFF image in the workspace folder with the same title.

- ALSO NOTE: AFTER THE APPERANCE OF A HISTOGRAM, another black white picture pops out ‘(B2) CHOOSE A REGION FOR BASAL DECLINE’ /// choose where, CAN FIRST CHECK IN IMAGEJ TO CHOOSE A SMALL REGION (10x10 pixel) on fluorophore loading area BUT NOT SHOWING Ca++ RISES, i.e., only show baseline declining (or if there is no declining, just choose a region showing no declining) /// this trace will be used to correct the declining, by letting the raw data time courses to divide it.

- Finally, a figure of declining trace appears.

- Two histograms are generated, one showing the spatial distribution of the distance-to-center of all response regions, and the distribution of region area (µm²). The images are saved in workspace as “B2_Distribution_Region_Area” //// Of note, the source data saved as ”B2_All_RegionArea”.

Get time-courses for all responsive regions

(Get temporal (-spatial) info for region responses)

2017-04-21: separate inter-corr for three phases (RESTING, STI, POST-STI)

- run “EB3_Pixel_Screening_temporal_SpontRe.m” to get temporal and spatio-temporal information about responses (regions), for spontaneous Ca++ responses.

- One parameter is the “skip_example = “ //// just for showing example traces; sometimes, there are too many traces, which can not be properly shown in Matlab figures; so can omit some, if set a number of 2 in here, meaning, only responses of region 2, 4, 6 … are shown, etc

- will get

- The time course of all responses (all regions), raster plot saved as a tiff image titled “B3_Raster_Plot_AllResponses” (of note: regions arranged according to their distance from the center, 1 means center, uppermost one showing the one most distant from the center.); source data saved as “B3_All_responses” that contains time courses for each region, and can be imported into Igor for plotting (BUT NEED USE ANOTHER IGOR ROUTINE …).

- Integral per minute of all response traces. Its distribution image (the histogram) saved as “B3_Distribution_Response_Integral”; the source data saved as ”B3_All_Responses_Integral” that can be imported into Igor, pooled together with other cells’ data to plot the final histogram where a proper bin size should be settled.  // NOTE, THE HISTOGRAM SHOWN HERE IS A TEMPORARY DISPLAY, FOR FORMAL HISTOGRAM (especially for pooled data, e.g., INTEGRAL DATA FROM SEVERAL RECORDINGS), NEED use B4 to generate the X and Y data points for a histogram and then replot in Igor (see below).

- The distribution of the correlation between all responses (combinations). THIS IS THE INDICATION ON TO WHICH EXTEND THE RESPONSES OF ALL REGIONS ARE SYNCHRONIZED OR NOT

The histogram saved as a tiff image titled “B3_Distribution_All_InterResponse_Corr”, source data saved as “B3_All_Responses_InterCorr”

// (170421) ALSO separate into three phases, in three txt files:

- B3_All_Responses_InterCorrREST

- B3_All_Responses_InterCorrSTI

- B3_All_Responses_InterCorrPS

- Example traces. The traces saved a tiff image “B3_Example_Response_Traces”.

Note that the example traces are taken every ‘skip_example’ regions from all responses (regions). // as the example traces are marked by the N° of region, the source data points can be easily found in “B3_All_Responses” in Igor.

- Relation between the correlation of responses and their clustering degree.  The correlation and the inter-distance (a reflection of clustering) between all combinations of all regions are calculated and scatter is plotted, showing whether closer regions are more correlated or not. // Image saved as “B3_Scatter_Inter_Response_Corr_vs_Inter_Distance” //

source data (i.e., X, Y axis data points) saved as “B3_Inter_Response_Corr.txt”, and “B3_Inter_Response_Distance.txt”, respectively

- Here for evoked recording, raw data points of three more integral results are shown:

- B3_Responses_Integral_resting.txt // integrals of single responses during resting period

- B3_Responses_Integral_sti.txt // integrals of single responses during stimulation

- B3_Responses_Integral_posti.txt // integrals of single responses after stimulation

Also, a figure is shown in Matlab and saved as tif 	“B3_Mean_Integral_from_Three_Phases”

(NEED NOTE, for global bath application of agonist, it’s not easy to do washing; so maybe, concretely, includes pre-stimulation phase (resting), and agonist presence phase (during and post-sti phase))

IMPORT MATLAB-GENERATED FILES INTO IGOR

- For each movie, create a subfolder under the root: In Igor, open ‘Data --> Data browser’, use New Folder function to create, then move the red ARROW pointing to the current subfolder

- Find & open the three procedures (i.e., small programs) in Igor: Windows  Other windows

  - ‘DAinput_170705bak.ipf’

  - ‘1_Igor_Load_Waves_transpose.ipf’

  - ‘Waterfall_plot.ipf’

- Use ‘DAinput_170705bak.ipf’ to import all generated data files from the Matlab workspace

  - In this procedure window, first change the 3rd line:

String pathname="D:\Dongdong\ZMwork"

Replace the current Matlab workspace folder pathway (i.e., the directory between “##”) with your proper workspace pathway directory

//*if can not modify the file, need first uncheck the read-only function, which locates at the bottom left corner, marked by a pensil sign.

  - Then, click the Compile command at the bottom left position.

  - Then, go to the general command window, a rectangle one usually located in the bottom of the Igor window, bearing the same name as the whole Igor file.

  - Input the procedure function name GAinput (contained by the procedure) in the command window (below the red line)

  - After entre key, the Matlab-generated data files can be automatically imported in the corresponding subfolder.

- Use ‘1_Igor_Load_Waves_transpose.ipf’ to import all response traces

  - In this procedure window, first change Matlab workspace pathway in the 5th line

String pathname="D:\Dongdong\ZMwork"

Replace the current Matlab workspace folder pathway (i.e., the directory between “##”) with your proper workspace pathway directory

//*if can not modify the file, need first uncheck the read-only function, which locates at the bottom left corner, marked by a pensil sign.

  - Then, change the movie length (i.e., how many frames in the movie)

Variable length_trace, roi, image_time=301

Need change 301 to your frame numbers

  - Then, go to the general command window, a rectangle one usually located in the bottom of the Igor window, bearing the same name as the whole Igor file.

  - Input the procedure function name P1() (contained by the procedure) in the command window (below the red line)

  - After entre key, the traces of all responses will be imported, named as ‘ROI_ROI_##’. A table automatically pops out. These traces can be directly plotted in one dimensional Graph, while all traces can also be plotted as a waterfall plot (see below).

- Use ‘Waterfall_plot.ipf’ procedure to plot all response traces (generated from one representative movie) into a waterfall plot

  - In this procedure window, first change the 3rd line

variable roi=35

This is the number of detected regions of interest (response regions)

Then change the 4th line

variable length_trace=301

This is the number of the image frames contained in the analyzed movie

//*if can not modify the file, need first uncheck the read-only function, which locates at the bottom left corner, marked by a pensil sign.

  - Then, go to the general command window, a rectangle one usually located in the bottom of the Igor window, bearing the same name as the whole Igor file.

  - Input the procedure function name waterfall_plot() (contained by the procedure) in the command window (below the red line)

  - After entre key, the traces of all responses will be plotted in a single waterfall plot

  - In the procedure, change the two parameters in

ModifyWaterfall angle=90, axlen=0.8

Can modify the appearance and view angle of the waterfall plot.

  - NOTE, after running this procedure, a 2D matrix containing all response traces will be automatically generated, named as ‘matrix_traces’. It can be used to plot other 2D figures in Igor: Windows  New  surface plot, image plot, Package New Waterfall, Polar plot, Box plot, etc

POOLED DATA (basic statistic) CHECK WITH MATLAB

Plot histogram with pooled data (B4)

- To plot a histogram for pooled data, all source data (not the Hist_x/y, but the source data used to generate the Hist_x/y) should be pooled together (e.g., in Matlab, just open the txt files containing the source data and paste them one by one in the [ ] in “B4”; or in Igor can put them together in a single column). // The histogram of the pooled data can then be plotted either in Igor, or in Matlab using “B4”

- In “B4”, the “bin_size” is set as a fraction of the full range of the pooled data, i.e., from 0 to 1.

- Then, insert the pooled data into the [] of “pooled_data = []”.

- Changing the bin_size until see a reasonable histogram.

- The result is saved as “Pooled_Hist_X/Y”, which can be imported into Igor for plotting.

Check the mean, and standard error or standard deviation (error bar)

- Can do this in Igor, choose the Column containing the pooled data, then

- Can also do this in Matlab:

- In the command window, create a new vector, e.g., : f>> q = [ COPY DATA TO HERE];

- Then ENTRÉE, then use “mean (q)” to get mean value, “std(q)” to get standard deviation, “std(q)/sqrt (length(q))” to get SE (standard error)

- Sd

Check if two groups of data are statistically different

- The two groups of data would appear as two columns in Igor. Let’s say, group1, and group2

- Then, in Matlab main window (command window, not the program window), can generate two vectors: e.g.,

>> q = [here put group1 data points];

>> c = [here put group2 data points]

- For Gaussian like distribution data (or for data of small number, i.e., hard to know the exact distribution), may just use ttest;

>> [h p i] = ttest2 (q, c)

(Then, can see p value // <0.05 would be significant)

- For data groups showing skewed distribution, can try Kolmogoro-Smirnov test;

>> [h p i] = kstest2 (q, c)

(Then, can see p value // <0.05 would be significant)