| Data Acquisition and Image Processing |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | HBc CLPs + Geraniol (2) |  | HBc CLP+ SLLGRM dimer (4) |  |  | HBc CLP + P1dC (7) |  |
| Microscope | Krios G3 300 kV, with Falcon III camera in linear mode |  |  |  |  |  |  |
| Illumination | Spot 5 nano probe; C2 aperture =70 µm; beam diameter 1 µm, |  |  |  |  |  |  |
| Imaging | Magnification: 75000 x, calibrated Pixel size: 1.064 Å/Px, objective aperture: 100 µm |  |  |  |  |  |  |
| Exposure | Total exposure = 40 e-/Å² in 20 fractions; exposure time: 2.6-3.2 s |  |  |  |  |  |  |
| Targeted underfocus | 600-1400 nm |  |  |  |  |  |  |
| Movies per stage position |  | 15 in 5 holes |  | 15 in 5 holes |  |  | 3 in 1 hole |
| Movies | 4875 |  | 4956 |  | 2784 |  |  |
| Template picked Particles | 331635 |  | 320845 |  | 36817 |  |  |
| Particles in final map | 229646 |  | 132610 |  | 22830 |  |  |
| Resolution Relion | 2.8 Å* |  | 2.8 Å* |  | 3.6 Å |  |  |
| Resolution CryoSparc | n/a |  | 2.5Å* |  | 3.0 Å |  |  |
|  | Models and Maps |  |  |  |  |  |  |
| MolProbity score | 1.61 |  | 1.58 |  | 1.43 |  |  |
| Clash score | 4.79 |  | 4.47 |  | 6.2 |  |  |
| Ramachandran Favored | 94.7% |  | 94.9% |  | 97.6% |  |  |
| Ramachandran Allowed | 5.3 % |  | 5.1% |  | 2.4% |  |  |
| Ramachandran Outliers | 0 % |  | 0% |  | 0% |  |  |
| Rotamer outliers | 0% |  | 0% |  | 0% |  |  |
| d FSC model (0/0.143/0.5) | 2.5Å/2.6Å/2.9Å |  | 2.1 Å /2.2 Å /2.7 Å |  | 2.6Å/2.7Å/3.2Å |  |  |
| Accession Codes | PDB: 8PWO EMD-17996 |  | PDB: 8PX6 EMD-18001 |  | PDB: 8PX3 EMD-18000 |  |  |